# Registers and Machine Cycles

To enhance the processing capability of a CPU, the ALU and control unit contain special storage areas called registers.  their function is to hold instructions, datavalues, and main memory addresses of both the instructions and data.  There are four basic types of registers.

- Instruction register – high-speed circuits within the control unit that hold an instruction to be executed.

- Address register – high-speed circuits within the control unit that hold the address of data to be processed or of the next instruction to be executed.

- Storage register – high-speed circuits most often in the ALU that temporarily store data retrieved form main memory prior to processing.

- Accumulator – high-speed circuits most often in the ALU that temporarily store the results of arithmetic and logic operations.

The processing of a single instruction occurs during a machine cycle.  It involves accessing data from main memory, using registers in the control unit and ALU to operate on that data, and transmitting results back to main memory.  A machine cycle consists of two parts:  an instruction cycle during which the control unit fetches the instruction from primary storage and prepares it for processing, and the execution cycle during which an instruction is actually executed and results produced.

Several basic operations are performed during the instruction cycle:

- The control unit fetches the instruction from main memory.

- The control unit interprets or decodes the instruction.

- The control unit places the part of the instruction that indicates the operation to bee performed in the instruction register and places the part of the instruction that indicates where the corresponding data is located in the address register.

The basic operations performed during the execution cycle are as follows:

- Using information in the address register, the control unit retrieves the data from main memory and places it in the storage register.

- Using the information in the instruction register, the control unit instructs the ALU to perform the required operation.

- The result of the operation is stored in an accumulator and then transmitted back to main memory.