SID Ruffian VHF Radio Procedures Card

| To use radio : - Engage brain before opening mouth - Switch on radio (connector + power switch on radio) - Select channel required - Adjust squelch to eliminate noise - Listen to ensure channel is not in use - Press transmit button on microphone - Keep message as brief as possible - Finish message with work OVER - Release transmit button on microphone and wait for reply | Channels: |  |
| --- | --- | --- |
|  | 16 | Strictly Distress, Urgency, Safety, Calling (max 60sec unless emergency) |
|  | 12 | Dublin Ship Port radio |
|  | 14 | Dun Laoghaire Ship Port radio |
|  | 6,8,10 | Ship to Ship |

PAN PAN procedures

- Use for vessel is in trouble but not in immediate danger (see examples below)

- Check radio is switched on (power lead and switch on radio)

- Select channel 16 High Power (25 watts)

- Press down and hold transmit button on microphone and say slowly and distictly :

PAN PAN            PAN PAN            PAN PAN

This is yacht RUFINIT RUFINIT RUFINIT

PAN PAN RUFINIT

MY POSITION IS   ....................  (location or bearing and distance from known point)

State nature of distress (e.g. broken rudder, not making way against tide/wind, lost engine power)

State type of help required/Number of people on board, any other relevant information

OVER  and wait for reply.